CSC-375 Final Project
Simplified C++ Group Chat System

Spring 2026 | Morehouse College | Operating Systems

# 1. Project Scope

This simplified version keeps the required folder layout and focuses on a working text-based group chat. The server supports multiple clients, active groups, group switching, recent message history, a thread pool, and a scheduling simulation. Optional audio/video is kept as a separate placeholder so the main project remains manageable.

# 2. Folder Layout

GroupChat/
├── client/
├── server/
├── shared/
├── tests/
├── logs/
├── diagrams/
├── README.md
├── CMakeLists.txt
└── final_report.docx

# 3. Required Features Covered

Clients can join or create groups using /join <group>.

Messages are broadcast to everyone in the same group.

Messages include timestamp, sender ID, and group name.

The server can manage multiple active groups.

A client can switch groups by joining a different group.

The /list command shows active groups.

Each group stores recent history in a fixed-size circular cache.

The server uses a thread pool with Round Robin or SJF task selection.

# 4. OS Concepts

## Threads and Synchronization

The server accepts network clients and submits message work into a fixed thread pool. Shared group state is protected using std::mutex. The thread pool uses std::condition_variable, which lets worker threads sleep while the task queue is empty.

## Scheduling

Round Robin mode uses FIFO order. SJF mode prioritizes shorter messages first by using message length as an estimated cost. This is an application-level scheduling simulation; the real operating system still schedules the actual CPU threads.

## Caching and Memory Limits

Each group owns a fixed-size circular message cache. When the cache is full, the oldest message is evicted. This models limited memory and gives new group members recent history without reading from disk.

# 5. How to Build and Run

mkdir build
cd build
cmake ..
make

./groupchat_server 5555 rr
./groupchat_client 127.0.0.1 5555

# 6. Diagrams

Thread architecture

Message flow

Virtual memory / cache simulation

Task queue scheduling

# 7. Simplification Decisions

The biggest simplification is using readable text commands instead of a fixed binary packet format. This makes debugging much easier while still demonstrating sockets, threads, synchronization, scheduling, and caching. The optional audio/video feature is represented by a stub file that can later be upgraded to send simulated binary chunks.